1. PRODUCT AND COMPANY NAME
PRODUCT CODE AND NAME
36307 SRI GREASE OEM NLGI 2
DESCRIPTION
Grease
COMPANY
CHEVRONTEXACO LTD
1 Westferry Circus, Canary Wharf
London E14 4HA 
U.K.
Tel : 0044/20 7719 3000
Fax : 0044/20 7719 5130
Emergency Phone Number : 0044/(0)18 65 407 333

_____________________________________________________________________________

2. COMPOSITION/INFORMATION ON INGREDIENTS

| Name | % Wt | CAS No. | EC No. |
| --- | --- | --- | --- |
| Distillates (petroleum), hydrotreated heavy paraffinic | 80 - 100 | 64742-54-7 | 265-157-1 |

_____________________________________________________________________________

(IP 346 DMSO extract < 3%)

_____________________________________________________________________________

3. HAZARDS IDENTIFICATION

| Product classification | Product is not classified as dangerous according to Directive 1999/45/EC. |
| --- | --- |
| Acute effects of exposure to man |  |
| Inhalation | Product not volatile at ambient temperature. |
|  | Irritating vapours or mist, causing discomfort in nose and throat, nasal discharge and coughing, may be generated as a result of particular work conditions. |
| Skin contact | Brief contact is not irritating. Prolonged contact, as with clothing wetted with material, may cause defatting of skin or irritation, seen as local redness with possible mild discomfort. |
|  | Material from high pressure equipment, pinhole leaks, or high pressure line failure can penetrate the skin and, if not properly treated, can cause severe injury, including disfigurement, loss of function, or even require amputation of the affectedarea. To prevent such serious injury, immediate medical attention should be sought even if the injection injury appears to be minor. |
| Eye contact | May cause minimal irritation, experienced as temporary discomfort. |
| Ingestion | No adverse effects expected. If more than several mouthfuls are swallowed, abdominal discomfort, nausea, and diarrhoea may occur. |
| Chronic effects of exposure to man |  |
| Medical conditions aggravated by exposure | Because of its defatting properties, prolonged and repeated skin contact may aggravate an existing dermatitis (skin condition). |

| Effects of exposure to the environment | Not expected to migrate due to viscosity. |
| --- | --- |

_____________________________________________________________________________

4. FIRST AID MEASURES

| Route of exposure |  |
| --- | --- |
| Inhalation | If the symptoms described above occur, remove to fresh air. |
| Skin contact | Wash skin with plenty of soap and water for several minutes. Get medical attention if skin irritation develops or persists. |
|  | High pressure injection of material into skin, if not properly treated, can cause severe injury. Most damage occurs during first few hours. Failure to debride the wound of all residual material can result in disfigurement, loss of function, or evenrequire amputation of the affected area. The extent of the injury cannot be determined by examining the wound. X-ray may reveal the presence of gas in tendon sheaths or soft tissue. |
| Eye contact | Flush eyes with plenty of water for several minutes. Get medical attention if eye irritation persists. |
| Ingestion | None considered necessary. |

_____________________________________________________________________________

5. FIRE-FIGHTING MEASURES

| Suitable extinguishing media | Use water fog, dry powder, foam or carbon dioxide. Use water to cool fire-exposed containers. If a leak or spill has not ignited, use water fog to disperse the vapours and to provide protection for personnel attempting to stop the leak. |
| --- | --- |
| Extinguishing media which must not be used for safety reasons | Water jet |
| Special exposure hazards arising from the substance or preparation itself, |  |
| combustion products, resulting gases | None |
| Special protective equipment for firefighters | The nature of special protective equipment required will depend upon the size of the fire, the degree of confinement of the fire and the natural ventilation available. Fire-resistant clothing and self-contained breathing apparatus is recommended forfires in confined spaces and poorly-ventilated areas. Full fire-proof clothing is recommended for any large fires involving this product. In case of fire - Always call the fire brigade. Small fires, such as those capable of being fought with ahand-held extinguisher, can normally be fought by a person who has received instruction on the hazards of flammable liquid fires. Fires that are beyond that stage should only be tackled by people who have received hands-on training. Ensure escape pathis available. |

_____________________________________________________________________________

6. ACCIDENTAL RELEASE MEASURES

| Procedures in case of accidental release or leakage | Avoid contact with skin and eyes. Remove mechanically. |
| --- | --- |

_____________________________________________________________________________

7. HANDLING AND STORAGE

| Handling | Avoid prolonged or repeated contact with skin. |
| --- | --- |
| Storage | Keep containers closed when not in use. Store at ambient temperature. |
| Specific use (s) | For intended product uses please refer to the Product Information Leaflet (PIL) |

_____________________________________________________________________________

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

| Respiratory protection | None normally required. |
| --- | --- |
| Hand and skin protection | Exposed employees should exercise reasonable personal cleanliness. This includes cleansing exposed skin areas several times daily with soap and water, and laundering or dry cleaning soiled work clothing. |
| Eye protection | Chemical type goggles or face shield recommended to prevent eye contact. |
| Exposure limit for the product | Oil mist, mineral (excluding metalworking fluids) : TWA : 5 mg/m3 ; STEL : 10 mg/m3 |

_____________________________________________________________________________

9. PHYSICAL AND CHEMICAL PROPERTIES

| Appearance | Green paste |
| --- | --- |
| Odour | Mineral oil |
| Flash point (ASTM D92), °C | > 200 |
| Relative density | 0.92 @ 15°C |
| Viscosity | Not applicable |

_____________________________________________________________________________

10. STABILITY AND REACTIVITY

| Materials to avoid | Strong oxidising agents. |
| --- | --- |
| Hazardous decomposition products | Oxides of carbon, aldehydes and ketones. |

_____________________________________________________________________________

11. TOXICOLOGICAL INFORMATION

| Acute |  |
| --- | --- |
| Inhalation | High concentrations of vapours or mist are likely to be irritating to the respiratory tract and may cause nausea, dizziness, headaches and drowsiness. |
| Skin contact | Slightly irritating to the skin. |
| Eye contact | Unlikely to cause more than transient stinging or redness if accidental eye contact occurs. |
| Ingestion | Unlikely to cause harm if accidentally swallowed in small doses, though larger quantities may cause nausea and diarrhoea. |

| Chronic | Repeated skin contact may cause a persistent irritation or dermatitis. |
| --- | --- |

_____________________________________________________________________________

12. ECOLOGICAL INFORMATION

| Mobility | Not expected to migrate due to viscosity. |
| --- | --- |
| Persistence and degradability | According to EC criteria : Not readily biodegradable |
| Potential to bioaccumulate | Considered unlikely to bioaccumulate. |
| Aquatic toxicity | Not classified as toxic. |
| Remarks | Believed not to represent a long-term danger to the aquatic environment. |

_____________________________________________________________________________

13. DISPOSAL CONSIDERATIONS

| Disposal | Dispose in accordance with local laws and regulations governing disposal of waste oil. |
| --- | --- |

_____________________________________________________________________________

14. TRANSPORT INFORMATION

| transport | Not regulated |
| --- | --- |

_____________________________________________________________________________

15. REGULATORY INFORMATION

| Classification/Labelling information | Under the criteria of Directive EEC/67/548 (dangerous substances) and EEC/1999/45 (dangerous preparations) : |
| --- | --- |
|  | Not classified |

_____________________________________________________________________________

16. OTHER INFORMATION

| other information | None |
| --- | --- |
| DATE ISSUED : 07/01/2004 | Supersedes : 07/01/2004 |

_____________________________________________________________________________

All information contained in this Material Safety Data Sheet and, in particular, the health and safety and environmental information is accurate to the best of our knowledge and belief as at the date of issue specified. However, the Company makes no warranty or representation, express or implied, as to the accuracy or completeness of such information.

The provision of this Material Safety Data Sheet is not intended, of itself, to obviate the need for all users to satisfy themselves that the product described is suitable for their individual purposes and that the safety precautions and environmental advice are adequate for their individual purposes and situation. Further, it is the user's obligation to use this product safely and to comply with all applicable laws and regulations concerning the use of the product.

The company accepts no responsibility for any injury, loss or damage, consequent upon any failure to follow the safety and other recommendations contained in this Material Safety Data Sheet, nor from any hazards inherent in the nature of the material, nor from any abnormal use of the material.

_____________________________________________________________________________

"Data sheet prepared by TEXACO BELGIUM N.V.

Technologiepark - Zwijnaarde 2 
B-9052 Gent / Zwijnaarde (Belgium) 
Tel. : +/32/9/240 7352 
Fax : +/32/9/240 7340"

_____________________________________________________________________________

Version nr : 1.12