--------------------------------------------------------------------------

644369-00 UNIREX N 3

MATERIAL SAFETY DATA BULLETIN

--------------------------------------------------------------------------

--------------------------------------------------------------------------

1. PRODUCT AND COMPANY IDENTIFICATION

--------------------------------------------------------------------------

PRODUCT NAME: UNIREX N 3

SUPPLIER: EXXONMOBIL OIL CORPORATION

3225 GALLOWS RD.

FAIRFAX,  VA  22037

24 - Hour Health and Safety Emergency (call collect):  609-737-4411

24 - Hour Transportation Emergency:

CHEMTREC: 800-424-9300   202-483-7616

LUBES AND FUELS: 281-834-3296

Product and Technical Information:

Lubricants and Specialties:  800-662-4525   800-443-9966

Fuels Products:  800-947-9147

MSDS Fax on Demand: 613-228-1467

MSDS Internet Website: http://emmsds.ihssolutions.com/

--------------------------------------------------------------------------

2. COMPOSITION/INFORMATION ON INGREDIENTS

--------------------------------------------------------------------------

CHEMICAL NAMES AND SYNONYMS: SEVERE TREAT MIN. OILS & ADDITIVES

GLOBALLY REPORTABLE MSDS INGREDIENTS:

Substance Name           Approx. Wt%

--------------           -----------

1-NAPHTHYLAMINE, N-PHENYL     <1

(90-30-2)

See Section  8 for exposure limits (if applicable).

--------------------------------------------------------------------------

3. HAZARDS IDENTIFICATION

--------------------------------------------------------------------------

Under normal conditions of use, this product is not considered hazardous

according to regulatory guidelines (See section 15).

EMERGENCY OVERVIEW: Green Grease.  DOT ERG No. :  NA

POTENTIAL HEALTH EFFECTS: Under normal conditions of intended use,

this product does not pose a risk to health.  Excessive exposure

may result in eye, skin or respiratory irritation.

POTENTIAL ENVIRONMENTAL EFFECTS: Harmful to aquatic organisms, may

cause long-term adverse effects in the aquatic environment.

For further health effects/toxicological data, see Section 11.

--------------------------------------------------------------------------

4. FIRST AID MEASURES

--------------------------------------------------------------------------

EYE CONTACT: Flush thoroughly with water.  If irritation occurs, call

a physician.

SKIN CONTACT: Wash contact areas with soap and water.  Remove and

clean oil soaked clothing daily and wash affected area.

INJECTION INJURY WARNING:  If product is injected into or under

the skin, or into any part of the body, regardless of the

appearance of the wound or its size, the individual should be

evaluated immediately by a physician as a surgical emergency.

Even though initial symptoms from high pressure injection may be

minimal or absent, early surgical treatment within the first few

hours may significantly reduce the ultimate extent of injury.

INHALATION: Remove from further exposure.  If respiratory irritation,

dizziness, nausea, or unconsciousness occurs, seek immediate

medical assistance.  If breathing has stopped, assist ventilation

with mechanical device or use mouth-to-mouth resuscitation.

INGESTION: Not expected to be a problem.  Seek medical attention if

discomfort occurs.  Do not induce vomiting.

--------------------------------------------------------------------------

5. FIRE-FIGHTING MEASURES

--------------------------------------------------------------------------

EXTINGUISHING MEDIA: Carbon dioxide, foam, dry chemical and water fog.

SPECIAL FIRE FIGHTING PROCEDURES: Water or foam may cause frothing.

Use water to keep fire exposed containers cool.  Water spray may

be used to flush spills away from exposure.  Prevent runoff from

fire control or dilution from entering streams, sewers, or

drinking water supply.

SPECIAL PROTECTIVE EQUIPMENT: For fires in enclosed areas, fire

fighters must use self-contained breathing apparatus.

UNUSUAL FIRE AND EXPLOSION HAZARDS: None.

COMBUSTION PRODUCTS: Fumes, smoke, carbon monoxide, sulfur oxides,

aldehydes and other decomposition products, in the case of

incomplete combustion.

Flash Point C(F): 200(392) (ESTIMATED FOR OIL, ASTM D-92 (COC)).

Flammable Limits (approx.% vol.in air) -   LEL: NE,   UEL: NE

NFPA HAZARD ID:   Health: 0,  Flammability: 1,  Reactivity: 0

--------------------------------------------------------------------------

6. ACCIDENTAL RELEASE MEASURES

--------------------------------------------------------------------------

NOTIFICATION PROCEDURES: Report spills/releases as required to

appropriate authorities.  U.S. Coast Guard and EPA regulations

require immediate reporting of spills/releases that could reach

any waterway including intermittent dry creeks.  Report

spill/release to Coast Guard National Response Center toll free

number (800)424-8802.  In case of accident or road spill notify

CHEMTREC (800) 424-9300.

PROCEDURES IF MATERIAL IS RELEASED OR SPILLED:

LAND SPILL: Shut off source taking normal safety precautions.  Take

measures to minimize the effects on ground water.  Recover by

pumping or contain spilled material with sand or other suitable

absorbent and remove mechanically into containers.  If necessary,

dispose of adsorbed residues as directed in Section 13.

WATER SPILL: Confine the spill immediately with booms.  Warn other

ships in the vicinity.  Notify port and other relevant authorities.

Remove from the surface by skimming or with suitable absorbents.  If

permitted by regulatory authorities the use of suitable dispersants

should be considered where recommended in local oil spill

procedures.

ENVIRONMENTAL PRECAUTIONS: Prevent material from entering sewers,

water sources or low lying areas; advise the relevant authorities

if it has, or if it contaminates soil/vegetation.

PERSONAL PRECAUTIONS: See Section 8

--------------------------------------------------------------------------

7. HANDLING AND STORAGE

--------------------------------------------------------------------------

HANDLING: High pressure injection under the skin may occur due to the

rupture of pressurized lines.  Always seek medical attention.  No

special precautions are necessary beyond normal good hygiene

practices.  See Section 8 for additional personal protection

advice when handling this product.

STORAGE: Keep containers closed when not in use.  Do not store in open

or unlabelled containers.  Store away from strong oxidizing

agents and combustible materials.  Do not store near heat,

sparks, flame or strong oxidants.

SPECIAL PRECAUTIONS: Prevent small spills and leakages to avoid slip

hazard.

EMPTY CONTAINER WARNING: Empty containers retain residue (liquid

and/or vapor) and can be dangerous. DO NOT PRESSURIZE, CUT, WELD,

BRAZE, SOLDER, DRILL, GRIND OR EXPOSE SUCH CONTAINERS TO HEAT,

FLAME, SPARKS, STATIC ELECTRICITY, OR OTHER SOURCES OF IGNITION;

THEY MAY EXPLODE AND CAUSE INJURY OR DEATH. Do not attempt to

refill or clean container since residue is difficult to remove.

Empty drums should be completely drained, properly bunged and

promptly returned to a drum reconditioner. All containers should

be disposed of in an environmentally safe manner and in

accordance with governmental regulations.

--------------------------------------------------------------------------

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

--------------------------------------------------------------------------

OCCUPATIONAL EXPOSURE LIMITS:

This product does not contain any components which have recognized

exposure limits.

VENTILATION: Use adequate ventilation.

RESPIRATORY PROTECTION: No special requirements under ordinary

conditions of use and with adequate ventilation.

EYE PROTECTION: Generally eye contact is unlikely with this type

material.  If eye contact is likely, safety glasses with side

shields or chemical type goggles should be worn.

SKIN PROTECTION: If prolonged or repeated skin contact is likely, oil

impervious gloves should be worn.  Good personal hygiene

practices should always be followed.

--------------------------------------------------------------------------

9. PHYSICAL AND CHEMICAL PROPERTIES

--------------------------------------------------------------------------

Typical physical properties are given below.  Consult Product Data Sheet

for specific details.

APPEARANCE: Grease

COLOR: Green

ODOR: Mild

ODOR THRESHOLD-ppm: NE

pH: NA

BOILING POINT C(F): NE

DROP POINT C(F): > 205(401)

FLASH POINT C(F): 200(392) (ESTIMATED FOR OIL, ASTM D-92 (COC))

FLAMMABILITY (solids): NE

AUTO FLAMMABILITY C(F): NA

EXPLOSIVE PROPERTIES: NA

OXIDIZING PROPERTIES: NA

VAPOR PRESSURE-mmHg 20 C: < 0.1

VAPOR DENSITY: NE

EVAPORATION RATE: NE

RELATIVE DENSITY, 15/4 C: 0.92

SOLUBILITY IN WATER: Negligible

PARTITION COEFFICIENT: > 3.5

VISCOSITY AT 40 C, cSt:   112.5

VISCOSITY AT 100 C, cSt:   12.0

POUR POINT C(F): NA

FREEZING POINT C(F): NE

VOLATILE ORGANIC COMPOUND: NE

NOTE: MOST PHYSICAL PROPERTIES FOR OIL COMPONENT.

DMSO EXTRACT, IP-346 (WT.%): <3, for mineral oil only

NA=NOT APPLICABLE NE=NOT ESTABLISHED D=DECOMPOSES

FOR FURTHER TECHNICAL INFORMATION, CONTACT YOUR MARKETING REPRESENTATIVE

--------------------------------------------------------------------------

10. STABILITY AND REACTIVITY

--------------------------------------------------------------------------

STABILITY (THERMAL, LIGHT, ETC.): Stable.

CONDITIONS TO AVOID: Extreme heat and high energy sources of ignition.

INCOMPATIBILITY (MATERIALS TO AVOID): Strong oxidizers.

HAZARDOUS DECOMPOSITION PRODUCTS: Product does not decompose at

ambient temperatures.

HAZARDOUS POLYMERIZATION: Will not occur.

--------------------------------------------------------------------------

11. TOXICOLOGICAL DATA

--------------------------------------------------------------------------

---ACUTE TOXICOLOGY---

ORAL TOXICITY (RATS):  Practically non-toxic (LD50:  greater than 2000

mg/kg).  ---Based on testing of similar products and/or the

components.

DERMAL TOXICITY (RABBITS):  Practically non-toxic (LD50:  greater than

2000 mg/kg).  ---Based on testing of similar products and/or the

components.

INHALATION TOXICITY (RATS):  Practically non-toxic (LC50:  greater

than 5 mg/l).  ---Based on testing of similar products and/or the

components.

EYE IRRITATION (RABBITS):  Practically non-irritating.  (Draize score:

greater than 6 but 15 or less).  ---Based on testing of similar

products and/or the components.

SKIN IRRITATION (RABBITS):  Practically non-irritating.  (Primary

Irritation Index:  greater than 0.5 but less than 3).  ---Based

on testing of similar products and/or the components.

OTHER ACUTE TOXICITY DATA: Although an acute inhalation study was not

performed with this product, a variety of mineral oils and

synthetic base oils, such as those in this product have been

tested.  These samples had virtually no effect other than a

nonspecific inflammatory response in the lung to the aerosolized

mineral oil.  The presence of additives in other tested

formulations (in approximately the same amounts as in the present

formulation) did not alter the observed effects.

---SUBCHRONIC TOXICOLOGY (SUMMARY)---

No significant adverse effects were found in studies using repeated

dermal applications of similar formulations to the skin of

laboratory animals for 13 weeks at doses significantly higher

than those expected during normal industrial exposure.  The

animals were evaluated extensively for effects of exposure

(hematology, serum chemistry, urinalysis, organ weights,

microscopic examination of tissues etc.).

---REPRODUCTIVE TOXICOLOGY (SUMMARY)---

No teratogenic effects would be expected from dermal exposure, based

on laboratory developmental toxicity studies of major components

in this formulation and/or materials of similar composition.

---CHRONIC TOXICOLOGY (SUMMARY)---

Repeated and/or prolonged exposure may cause irritation to the skin,

eyes or respiratory tract.  For mineral base oils:  Base oils in

this product are severely solvent refined and/or severely

hydrotreated.  Chronic mouse skin painting studies of severely

treated oils showed no evidence of carcinogenic effects.  These

results are confirmed on a continuing basis using various

screening methods such as Modified Ames Test, IP-346, and/or

other analytical methods.  For synthetic base oils:  The base

oils in this product have been tested in the Ames assay and other

tests of mutagenicity with negative results.  These base oils are

not expected to be carcinogenic with chronic dermal exposures.

---SENSITIZATION (SUMMARY)---

Not expected to be sensitizing based on tests of this product,

components, or similar products.

--------------------------------------------------------------------------

12. ECOLOGICAL INFORMATION

--------------------------------------------------------------------------

ENVIRONMENTAL FATE AND EFFECTS:

This environmental assessment was conducted using information on the

individual components as no test data was available for this

specific formulation.

ECOTOXICITY: This product is expected to be harmful to aquatic

organisms. May cause long-term adverse effects in the aquatic

environment.

MOBILITY: Not established.

PERSISTENCE AND DEGRADABILITY: This product is expected to be

inherently biodegradable, as the principal components have been

shown to degrade at slow to moderate rates.

BIOACCUMULATIVE POTENTIAL: Not established.

--------------------------------------------------------------------------

13. DISPOSAL CONSIDERATIONS

--------------------------------------------------------------------------

WASTE DISPOSAL: Product is suitable for burning in an enclosed,

controlled burner for fuel value.  Such burning may be limited

pursuant to the Resource Conservation and Recovery Act.  In

addition, the product is suitable for processing by an approved

recycling facility or can be disposed of at an appropriate

government waste disposal facility.  Use of these methods is

subject to user compliance with applicable laws and regulations

and consideration of product characteristics at time of disposal.

RCRA INFORMATION:  Disposal of unused product may be subject to RCRA

regulations (40 CFR 261).  Disposal of the used product may also

be regulated due to ignitability, corrosivity, reactivity, or

toxicity as determined by the Toxicity Characteristic Leaching

Procedure (TCLP).

BARIUM:    .0677 PCT (TCLP)

--------------------------------------------------------------------------

14. TRANSPORT INFORMATION

--------------------------------------------------------------------------

USA DOT:  NOT REGULATED BY USA DOT.

RID/ADR:  NOT REGULATED BY RID/ADR.

IMO:  NOT REGULATED BY IMO.

IATA:  NOT REGULATED BY IATA.

--------------------------------------------------------------------------

15. REGULATORY INFORMATION

--------------------------------------------------------------------------

US OSHA HAZARD COMMUNICATION STANDARD: When used for its intended

purposes, this product is not classified as hazardous in

accordance with OSHA 29 CFR 1910.1200.

EU Labeling: Product is dangerous as defined by the European Union

Dangerous Substances/Preparations Directives.

Symbol: Not applicable.

Risk Phrase(s): R52/53.

Harmful to aquatic organisms, may cause long-term adverse effects in

the aquatic environment.

Safety Phrase(s): S61.

Avoid release to the environment.  Refer to special

instructions/Safety data sheets.

Governmental Inventory Status: All components comply with TSCA,

EINECS/ELINCS, and DSL.

U.S. Superfund Amendments and Reauthorization Act (SARA) Title III:

This product contains no "EXTREMELY HAZARDOUS SUBSTANCES".

SARA (311/312) REPORTABLE HAZARD CATEGORIES: None.

This product contains no chemicals subject to the supplier notification

requirements of SARA (313) toxic release program.

The following product ingredients are cited on the lists below:

CHEMICAL NAME                         CAS NUMBER     LIST CITATIONS *

-------------                         ----------     --------------

LITHIUM HYDROXIDE   (0.35%)            1310-65-2     22

XYLENES   (0.05%)                      1330-20-7     22

LITHIUM-SOAP THICKENER   (12.75%)      7620-77-1     22

LITHIUM METABORATE   (0.94%)          13453-69-5     22

FATTY ACIDS, C16-22, LITHIUM SALTS    68783-36-8     22

(1.58%)

--- REGULATORY LISTS SEARCHED ---

1=ACGIH ALL  6=IARC 1     11=TSCA 4    16=CA P65 CARC   21=LA RTK

2=ACGIH A1   7=IARC 2A    12=TSCA 5a2  17=CA P65 REPRO  22=MI 293

3=ACGIH A2   8=IARC 2B    13=TSCA 5e   18=CA RTK        23=MN RTK

4=NTP CARC   9=OSHA CARC  14=TSCA 6    19=FL RTK        24=NJ RTK

5=NTP SUS   10=OSHA Z     15=TSCA 12b  20=IL RTK        25=PA RTK

26=RI RTK

* EPA recently added new chemical substances to its TSCA Section 4 test rules.  Please contact the supplier to confirm whether the ingredients in this product currently appear on a TSCA 4 or TSCA 12b list.

Code key:CARC=Carcinogen; SUS=Suspected Carcinogen; REPRO=Reproductive

--------------------------------------------------------------------------

16. OTHER INFORMATION

--------------------------------------------------------------------------

USE: GREASE

NOTE: PRODUCTS OF EXXON MOBIL CORPORATION AND ITS AFFILIATED COMPANIES

ARE NOT FORMULATED TO CONTAIN PCBS.

Health studies have shown that many hydrocarbons pose potential human

health risks which may vary from person to person. Information provided

on this MSDS reflects intended use. This product should not be used for

other applications. In any case, the following advice should be

considered:

INDUSTRIAL LABEL

Under normal conditions of intended use, this product does not pose a

risk to health.  Excessive exposure may result in eye, skin or

respiratory irritation.  Always observe good hygiene measures.  First

Aid: Wash skin with soap and water.  Flush eyes with water.  If

overcome by fumes or vapor, remove to fresh air.  If ingested do not

induce vomiting.  If symptoms persist seek medical assistance.  Read

and understand the MSDS before using this product.

*************************************************************************

For Internal Use Only:  MHC:  1* 1* 1* 1* 1*, MPPEC:  A, TRN:  644369-00,

CMCS97:  97N712, REQ:  US - MARKETING, SAFE USE:  L

EHS Approval Date: 20OCT2003

*************************************************************************

Information given herein is offered in good faith as accurate, but

without guarantee. Conditions of use and suitability of the product for

particular uses are beyond our control; all risks of use of the product

are therefore assumed by the user and WE EXPRESSLY DISCLAIM ALL

WARRANTIES OF EVERY KIND AND NATURE, INCLUDING WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE IN RESPECT TO THE

USE OR SUITABILITY OF THE PRODUCT. Nothing is intended as a

recommendation for uses which infringe valid patents or as extending

license under valid patents. Appropriate warnings and safe handling

procedures should be provided to handlers and users. Alteration of this

document is strictly prohibited. Except to the extent required by law,

republication or retransmission of this document, in whole or in part, is

not permitted. Exxon Mobil Corporation and its affiliated companies

assume no responsibility for accuracy of information unless the document

is the most current available from an official ExxonMobil distribution

system. Exxon Mobil Corporation and its affiliated companies neither

represent nor warrant that the format, content or product formulas

contained in this document comply with the laws of any other country

except the United States of America.

Prepared by:  ExxonMobil Oil Corporation

Environmental Health and Safety Department, Clinton, USA